AI Chip Center for Emerging Smart Systems

Project: CoDesign

Micro-Architecture Document: XXXX Version 0.1

Abstract: This document provides detailed descriptions of function spec, micro-architecture, interface signals, config register and protocols of the xxxx module in CoDesign ASIC project.

Confidential

# Revision History

| Version | Date | Author(s) | Description |
| --- | --- | --- | --- |
| 0.1 | July.07, 2022 | xxxx | Initial draft |

Contents

Revision History	2

1. Overview	4

1.1 Function List	4

1.2 Acronyms	5

2. I/O Signal List	6

3. Timing Diagram and Configuration Register	8

3.1 BUF_MGR timing diagram	8

3.2 GPIF timing diagram	8

3.3 Configuration Registers	9

3.3.1 Register Definition	9

3.3.2 Register Description	10

4. Module Micro-architecture	12

4.1 GPIF2MMGR (Rx Port)	12

4.2 MMGR2GPIF (Tx Port)	13

4.3 XXXX	13

# 1. Overview

The Function of IB module is to control the transition between DDR MMGR/BUF_MGR interface and GPIF interface. GPIF interface is a FIFO interface for data transaction. The IB module’s block diagram and connection with other modules is shown in Figure 1-1.

Figure 1-1.  IB block Diagram

## 1.1 Function List

- IO port physical feature

  - IO port typical frequency 50Mhz, up to 100MHz;

  - One set of GPIF input port;

  - One set of GPIF output port;

- Block size and bandwidth

  - Support configurable data block size (FX3 firmware may require the packet size should be multiples of 1KB, range 1~6172 KB);

  - Support bandwidth up to 150MB/s with maximum block size;

- Data Transform

  - Support MMGR port to GPIF port protocol transform;

  - Support GPIF port to MMGR port protocol transform;

  - Cross clock via async FIFOs

- Loop back scheme for testability

  - Support GPIF interface loop back test;

  - Support MMGR interface loop back test;

- Error detect

  - Support FIFOs underflow and overflow detection;

  - Support CRC check in GPIF interface;

## 1.2 Acronyms

| Acronyms | Meaning |
| --- | --- |
| APB | AMBA Peripheral Bus |
| BM | Boot mode: 0: DDR mode; 1: Non-DDR mode |
| GPIF | General Purpose Interface |
| MMGR | Memory management |
| I2CS | I2C slave |
| CHIP_CTRL/ CCTRL | Chip Controller |
| IB | Interface Block |
| CCODE | Configure Code field in each package |
| DB | Data Block field in each package |

# 2. I/O Signal List

| Signal Name | I/O | Bit Width | Descriptions |
| --- | --- | --- | --- |
| Global signals |  |  |  |  |  |  |
| clk | Input | 1 | IB system clock |
| rst_n | Input | 1 | reset input, active-low |
| cfg_rst_n | Input | 1 | Configure reset, active-low |
| gpif_in_clk | Input | 1 | GPIF Rx port clock |
| gpif_out_clk | Input | 1 | GPIF Tx port clock |
| gpif_in_rst_n | Input | 1 | GPIF input clock domain reset, active low |
| gpif_out_rst_n | Input | 1 | GPIF output clock domain reset, active low |
| ddr_mode_sel | Input | 1 | 1: DDR mode; 0: Non-DDR mode |
| APB Slave Signals |  |  |  |  |  |
| apb_s_psel | Input | 1 | APB3 slave select |
| apb_s_penable | Input | 1 | APB3 slave enable |
| apb_s_paddr | Input | 16 | APB3 slave address bus |
| apb_s_pwrite | Input | 1 | APB3 slave write/read |
| apb_s_pwdata | Input | 32 | APB3 slave write data bus |
| apb_s_pready | Output | 1 | APB3 slave ready |
| apb_s_prdata | Output | 32 | APB3 slave read data bus |
| apb_s_pslverr | Output | 1 | APB3 slave error |
| Interface with DDR MMGR read bus |  |  |  |  |  |
| mmgr_ib_rd_ack | Input | 1 | DDR_MGR read acknowledge |
| mmgr_ib_rd_vld | Input | 1 | DDR_MGR read data valid |
| mmgr_ib_rd_data | Input | 256 | DDR_MGR read data bus |
| mmgr_ib_rd_end | Input | 1 | DDR_MGR read data end |
| ib_mmgr_rd_req | Output | 1 | DDR_MGR read request |
| ib_mmgr_rd_size | Output | 10 | DDR_MGR read size; 0 means data size is 1 |
| ib_mmgr_rd_addr | Output | 26 | DDR_MGR read address |
| Interface with DDR MMGR write bus |  |  |  |  |  |
| mmgr_ib_wr_ack | Input | 1 | DDR_MGR write acknowledge |
| mmgr_ib_wr_rdy | Input | 1 | DDR_MGR write ready |
| ib_mmgr_wr_req | Output | 1 | DDR_MGR write request |
| ib_mmgr_wr_size | Output | 10 | DDR_MGR write size; 0 means data size is 1 |
| ib_mmgr_wr_vld | Output | 1 | DDR_MGR write enable |
| ib_mmgr_wr_addr | Output | 26 | DDR_MGR write address bus |
| ib_mmgr_wr_data | Output | 256 | DDR_MGR write data bus |
| ib_mmgr_wr_end | Output | 1 | DDR_MGR write data end |
| Interface with BUF_MGR read bus |  |  |  |  |  |  |
| bufmgr_ib_rd_ack | Input | 1 | BUF_MGR read acknowledge |
| bufmgr_ib_rd_vld | Input | 1 | BUF_MGR read data valid |
| bufmgr_ib_rd_data | Input | 256 | BUF_MGR read data bus |
| bufmgr_ib_rd_end | Input | 1 | BUF_MGR read data end |
| ib_bufmgr_rd_req | Output | 1 | BUF_MGR read request |
| ib_bufmgr_rd_size | Output | 8 | BUF_MGR read size; 0 means data size is 1 |
| ib_bufmgr_rd_addr | Output | 16 | BUF_MGR read address |
| Interface with BUF_MGR write bus |  |  |  |  |
| bufmgr_ib_wr_ack | Input | 1 | BUF_MGR write acknowledge |
| bufmgr_ib_wr_rdy | Input | 1 | BUF_MGR write ready |
| ib_bufmgr_wr_req | Output | 1 | BUF_MGR write request |
| ib_bufmgr_wr_size | Output | 8 | BUF_MGR write size; 0 means data size is 1 |
| ib_bufmgr_wr_vld | Output | 1 | BUF_MGR write enable |
| ib_bufmgr_wr_addr | Output | 16 | BUF_MGR write address bus |
| ib_bufmgr_wr_data | Output | 256 | BUF_MGR write data bus |
| ib_bufmgr_wr_end | Output | 1 | BUF_MGR write data end |
| Interface with Chip_Ctrl (frame_in_fifo) |  |  |  |  |  |  |
| fmi_fifo_afull | Input | 1 | Frame in FIFO almost full flag |
| fmi_fifo_wr | Output | 1 | Frame in FIFO write enable |
| fmi_fifo_wdata | Output | 73 | Frame in FIFO write data bus [72]: init_end [71]: init_start [70]: init_marker [69:68]: frame_cnt [67:52]: out_img_step [51:32]: db_out_size [31:24]: frame_id [23:0]:db_out_addr |
| fmi_fifo_eof | Output | 1 | Frame input end of frame indicator |
| Interface with Chip_Ctrl (frame_out_fifo) |  |  |  |  |  |
| fmo_fifo_empty | Input | 1 | Frame out FIFO empty flag |
| fmo_fifo_rdata | Input | 176 | Frame out FIFO write data bus [175:70]: chip_ctrl status [69:68]: frame_cnt [67:52]: out_img_step [51:32]: db_out_size [31:24]: frame_id [23:0]: db_out_addr |
| fmo_fifo_rd | Output | 1 | Frame out FIFO read enable |
| fmo_eof | Output | 1 | Frame out end of frame indicator |
| Interface with Chip_Ctrl (Header Buffer) |  |  |  |  |  |
| ib_cctrl_hd_wen | Output | 1 | Header Buffer write enable |
| ib_cctrl_hd_waddr | Output | 3 | Header Buffer write address |
| ib_cctrl_hd_wdata | Output | 256 | Header Buffer write data |
| GPIF Input Interface |  |  |  |  |  |  |
| gpif_in_data | Input | 16 | GPIF input port data bus |
| gpif_in_aemp | Input | 1 | GPIF input port almost empty flag, water marker with 12Byte data remain, active low 1’b0: almost empty 1’b1: ready to read |
| gpif_in_dma | Input | 1 | GPIF input DMA ready |
| gpif_in_csn | Output | 1 | GPIF input port chip select, active low |
| gpif_in_oen | Output | 1 | GPIF input port output enable, active low |
| gpif_in_rdn | Output | 1 | GPIF input port read enable, active low |
| GPIF Output Interface |  |  |  |  |
| gpif_out_aful | Input | 1 | GPIF output port almost full flag, water marker with 12Byte space remain, active low 1'b0: almost full 1'b1: ready to write |
| gpif_out_dma | Input | 1 | GPIF output DMA ready |
| gpif_out_csn | Output | 1 | GPIF output port chip select, active low |
| gpif_out_wrn | Output | 1 | GPIF output port write enable, active low |
| gpif_out_pen | Output | 1 | GPIF output port packet end, active low |
| gpif_out_data | Output | 16 | GPIF output port data bus |

# 3. Timing Diagram and Configuration Register

## 3.1 BUF_MGR timing diagram

IB receives header and instructions packet from Host and sends to on-chip BUF_MGR.

In Non-DDR mode, IB also receives input frame data packet from Host and sends data to BUF_MGR. For frame result, IB read data from BUF_MGR and sends to external Host.

BUF_MGR write timing

BUF_MGR read timing

## 3.2 GPIF timing diagram

GPIF read timing

GPIF write timing

## 3.3 Configuration Registers

### 3.3.1 Register Definition

| Offset | Name | Mnemonic | Reset Value |
| --- | --- | --- | --- |
| 0x000 | GPIF Status Register | GPIF_STATUS | 0x0 |
| 0x004 | GPIF Preamble field size | GPIF_PRMB_SIZE | 0xF |
| 0x008 | GPIF RD/WR MMGR maximum size | GPIF_MSIZE | {0xF, 0xF} |
| 0x00C | GPIF Control Register | GPIF_CTRL | 0x0 |
| 0x010 | GPIF Threshold Register | GPIF_TH | 0x0 |
| 0x014 | GPIF Frame Queue | FRAME_QUEUE | 0x0 |
| 0x018 | GPIF Preamble double word 0 | GPIF_PRMB_DW00 | 0x01234567 |
| 0x01C | GPIF Preamble double word 1 | GPIF_PRMB_DW01 | 0x89abcdef |
| 0x020 | GPIF Preamble double word 2 | GPIF_PRMB_DW02 | 0x76543210 |
| 0x024 | GPIF Preamble double word 3 | GPIF_PRMB_DW03 | 0xfedcba98 |
| 0x028 | GPIF Preamble double word 4 | GPIF_PRMB_DW04 | 0x01234567 |
| 0x02C | GPIF Preamble double word 5 | GPIF_PRMB_DW05 | 0x89abcdef |
| 0x030 | GPIF Preamble double word 6 | GPIF_PRMB_DW06 | 0x76543210 |
| 0x034 | GPIF Preamble double word 7 | GPIF_PRMB_DW07 | 0xfedcba98 |
| 0x038 | GPIF Status Register 1 | GPIF_STATUS1 | 0x0 |
|  | Reserved | Reserved |  |

### 3.3.2 Register Description

- GPIF_STATUS									offset 0x000

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:18 | RO | 0x0 | Reserved |
| crc16_data_err | 17 | W1C | 0x0 | gpif input data CRC16 check error |
| crc16_head_err | 16 | W1C | 0x0 | gpif input head CRC16 check error |
| gpif_cfifo_rerr | 15 | W1C | 0x0 | gpif configure FIFO read error |
| gpif_cfifo_werr | 14 | W1C | 0x0 | gpif configure FIFO write error |
| gpif_lfifo_rerr | 13 | W1C | 0x0 | gpif loopback data FIFO read error |
| gpif_lfifo_werr | 12 | W1C | 0x0 | gpif loopback data FIFO write error |
| mmgr_cfifo_rerr | 11 | W1C | 0x0 | mmgr configure FIFO read error |
| mmgr_cfifo_werr | 10 | W1C | 0x0 | mmgr configure FIFO write error |
| mmgr_lfifo_rerr | 9 | W1C | 0x0 | mmgr loopback data FIFO read error |
| mmgr_lfifo_werr | 8 | W1C | 0x0 | mmgr loopback data FIFO write error |
| mmgr2gpif_cfifo_rerr | 7 | W1C | 0x0 | mmgr2gpif configure FIFO read error |
| mmgr2gpif_cfifo_werr | 6 | W1C | 0x0 | mmgr2gpif configure FIFO write error |
| mmgr2gpif_dfifo_rerr | 5 | W1C | 0x0 | mmgr2gpif data FIFO read error |
| mmgr2gpif_dfifo_werr | 4 | W1C | 0x0 | mmgr2gpif data FIFO write error |
| gpif2mmgr_cfifo_rerr | 3 | W1C | 0x0 | gpif2mmgr configure FIFO read error |
| gpif2mmgr_cfifo_werr | 2 | W1C | 0x0 | gpif2mmgr configure FIFO write error |
| gpif2mmgr_dfifo_rerr | 1 | W1C | 0x0 | gpif2mmgr data FIFO read error |
| gpif2mmgr_dfifo_werr | 0 | W1C | 0x0 | gpif2mmgr data FIFO write error |

- GPIF_PRMB_SIZE 									offset 0x004

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:5 | RO | 0x0 | Reserved |
| prmb_size | 4:0 | RW | 0xF | GPIF preamble size -1, 16bit based. Preamble size = (gpif_prmb_size+1) *16bit |

- GPIF_MSIZE 									offset 0x008

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:20 | RO | 0x0 | Reserved |
| mmgr_rd_msize | 19:10 | RW | 0xF | MMGR read maximum size -1 |
| mmgr_wr_msize | 9:0 | RW | 0xF | MMGR write maximum size -1 |

- GPIF_CTRL   									 offset 0x00C

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:8 | RO | 0x0 | Reserved |
| crc16_data_adden | 7 | RW | 0x0 | CRC16 Data part add enable |
| crc16_head_adden | 6 | RW | 0x0 | CRC16 Head part add enable |
| crc16_data_chken | 5 | RW | 0x0 | CRC16 Data part check enable |
| crc16_head_chken | 4 | RW | 0x0 | CRC16 Head part check enable |
| Reserved | 3:2 | RO | 0x0 | Reserved |
| mmgr_lpb_en | 1 | RW | 0x0 | MMGR interface loopback enable |
| gpif_lpb_en | 0 | RW | 0x0 | GPIF interface loopback enable |

- GPIF_TH   										     offset 0x010

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:13 | RO | 0x0 | Reserved |
| gpif_aful_th | 12:8 | RW | 0x0 | GPIF output port almost full threshold, actual value will be (gpif_aful_th+4) |
| Reserved | 7:5 | RO | 0x0 | Reserved |
| gpif_aemp_th | 4:0 | RW | 0x0 | GPIF input port almost empty threshold, actual value will be (gpif_aemp_th+4) |

- FRAME_QUEUE									     offset 0x014

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:10 | RO | 0x0 | Reserved |
| out_queue_depth | 9:8 | RW | 0x0 | Output frame queue depth -1 |
| Reserved | 7:2 | RO | 0x0 | Reserved |
| in_queue_depth | 1:0 | RW | 0x0 | Input frame queue depth -1 |

- GPIF_PRMB_DW00								               offset 0x018

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw00 | 31:0 | RW | 0x01234567 | Preamble double word 0 |

- GPIF_PRMB_DW01								               offset 0x01C

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw01 | 31:0 | RW | 0x89abcdef | Preamble double word 1 |

- GPIF_PRMB_DW02								               offset 0x020

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw02 | 31:0 | RW | 0x76543210 | Preamble double word 2 |

- GPIF_PRMB_DW03								               offset 0x024

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw03 | 31:0 | RW | 0xfedcba98 | Preamble double word 3 |

- GPIF_PRMB_DW04								               offset 0x028

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw04 | 31:0 | RW | 0x01234567 | Preamble double word 4 |

- GPIF_PRMB_DW05								               offset 0x02C

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw05 | 31:0 | RW | 0x89abcdef | Preamble double word 5 |

- GPIF_PRMB_DW06								               offset 0x030

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw06 | 31:0 | RW | 0x76543210 | Preamble double word 6 |

- GPIF_PRMB_DW07								               offset 0x034

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| prmb_dw07 | 31:0 | RW | 0xfedcba98 | Preamble double word 7 |

- GPIF_STATUS1 									offset 0x038

| Field Name | Bits | R/W | Default | Description |
| --- | --- | --- | --- | --- |
| Reserved | 31:4 | RO | 0x0 | Reserved |
| out_frame_cnt | 3:2 | RO | 0x0 | Output frame counter |
| in_frame_cnt | 1:0 | RO | 0x0 | Input frame counter |

# 4. Module Micro-architecture

## 4.1 GPIF2MMGR (Rx Port)

Based on the DDR or non-DDR mode selection, the MMGR could be DDR_MGR or BUF_MGR.

For GPIF2MMGR direction, host will send frame through GPIF interface based on the frame format. The GPIF2MMGR module will do the steps:

Figure 4-1 GPIF2MMGR process schematic

1st step: gpif2fifo module will search the preamble. When the preamble been locked, it will jump to state S_PRE. In this state, it will extract data been pre-read in preamble locking state. Then it jumps to S_CFG, extract the remain configure head for whole 256bit and push the bit[223:0] into Rx_CFIFO (bit[255:224] is not used). If the CRC16 function is switched on, it will process CRC16 check with the last 16bit data of configure code.

2nd step: based on the configure db_in_size, gpif2fifo will read the correspond size data from GPIF interface. The gpif2fifo will packet the data with 256bit width and push it into Rx_DFIFO. If the CRC16 function is switched on, it will process CRC16 check with the last 16bit data of data block.

3rd step: when the Rx_CFIFO is no empty, fifo2mmgr module will read configure data from Rx_CFIFO. Meanwhile, if the fmi_fifo_afull is de-assert which means there is enough space for data saving, fifo2mmgr will jump to 4th step, otherwise it will jump to 6th step.

4th step: when the Rx_DFIFO has enough data to transmit to MMGR, fifo2mmgr will request MMGR_TOP for data transmission. When the MMGR write is been accept by MMGR_TOP, fifo2mmgr will read data from Rx_DFIFO and transmit to MMGR_TOP;

5th step: based on the configure data read from Rx_CFIFO, MMGR will transmit db_in_size data to MMGR_TOP and generate fmi_eof pulse when all the data is transmitted.

6th step: when the fmi_fifo_afull is assert which means there is no space for data saving, fifo2mmgr will wait for fmi_fifo_afull been de-assert, meanwhile backpressure gpif2fifo.

## 4.2 MMGR2GPIF (Tx Port)

For MMGR2GPIF direction, CHIP_CTRL will trigger the transmission by fulfill the FMO_FIFO. The mmgr2gpif module will transmit the frame to host through GPIF interface. The MMGR2GPIF module will do the steps:

Figure 4-4 MMGR2GPIF process schematic

1st step: when fmo_fifo_empty is de-assert, which means there are several data blocks need to transmit to host. The mmgr2fifo module will read configure data from FMO_FIFO and write it into Tx_CFIFO to acknowledge fifo2gpif that there are data blocks to transmit.

2nd step: when the Tx_DFIFO has enough space for data saving, mmgr2fifo module will request MMGR_TOP for data transmission. When the MMGR read is been accept by MMGR_TOP. mmgr2fifo will read data from MMGR_TOP and push it into Tx_DFIFO.

3rd step: when the Tx_CFIFO and Tx_DFIFO are both non-empty, fifo2gpif module will read configure head from Tx_CFIFO, and start GPIF interface; fifo2gpif module will read data from Tx_DFIFO when gpif_de is valid.

4th step: based on db_out_size, mmgr2fifo will read enough data from MMGR_TOP and write enough into Tx_DFIFO, while fifo2gpif will read these data and packet them into gpif_data bus.

5th step: during transmission period, CRC16 will be calculated automatically. If the CRC16 add is been switched on, the last 16bit of configure head and data block will be replace by the output of CRC16.

## 4.3 XXXX

[Other sub-modules’ micro-architecture and memory buffer usage & data format description]